# Worcester Polytechnic Institute

# Robotics Engineering Program

# Final Project Report

Submitted By
Joel Sutherland
Adam Panzica

Date Submitted		: 3/5/2010

Date Completed	: 3/5/2010

Course Instructor	: Profs. Fischer & Ciaraldi

Lab Section		: C02

# Abstract

A two-link arm was programmed to function as a pick-n-place robot, capable of locating and acquiring items on a moving assembly line and sort them by weight.

Contents

Abstract	1

Table of Figures	3

Introduction	4

Methodology	6

Results and Analysis	8

Discussion	8

Appendix A: Final_Project.h	10

Appendix B: Final_Project.c	15

# Table of Figures

Figure 1: Model for Theta2 Calculation	5

Figure 2: Theta2 Calculations	5

# Introduction

Assembly-line robots form the backbone of many modern industries.  Such robots must complete complex tasks consistently and efficiently for long periods of time.  For this project, a simple two-link “pick-n-place” robot was developed and programmed to perform a sorting task on items travelling down a conveyer belt.

We decided to use two Infrared Rangefinders to determine the position of the each box on the belt.  The first would determine the horizontal distance from the base of the arm, and a second would serve as a trigger, telling the robot that the box has reached the plane of the arm and van be lifted.  The arm would then calculate the position using Inverse Kinematics and pick it up using an electromagnet.  The arm then rose to a neutral position and determined the weight of the box based on motor torques and placed it in the appropriate container before returning to a neutral position.

The final product completed its task, but was quite inconsistent.  These issues will be discussed in full as part of the DISCUSSION section.  We encountered many sources of error, some of which we were unable to account for before the demonstration of the project.

# Methodology

To represent the system at a given time, we developed structs DATA_PACK  and JOINT which held information on the joint position and relevant sensor data, and various flags to represent the stage of the action.

The main program loop calls a State Machine which moves through states 0 through 3.  Stage 0 is has the arm in a neutral state, waiting for input.  As soon as an object is detected in range by the first IR Sensor, a reading is made and 16 sample values are averaged to determine the correct position on the belt.  This is done by the Check_For_Object function.  The arm then moves to State 1, hovering above the projected location of the box.  The Check_In_Range function then looks for when the box reaches the IR Sensor just before the plane of the arm.  When that is triggered, the arm moves to State 2, turning on the magnet and moving to the location of the box.  In State 4, the arm moves back up, summing Rsense values and using that data to sort.  The arm moves above the correct container and disables the magnet.

A function Calc_Inv_Kin was developed which performed the inverse kinematics calculations.  The joint angle of Joint 0 was simple, as it was laid out the same as the basic two-link arm studied in class.  The second joint value was calculated as described below.

Figure 1: Model for Theta2 Calculation

Figure 2: Theta2 Calculations

All values were stored as floats, and all calculations were floating-point calculations.  All calculations were done in radians and translated to degrees to send the target angles to the joint structures for PID control.

# Results and Analysis

The system was able to complete the task, but not with much consistency.  Several problems arose and were not remedied in time for demonstration and these are covered in the DISCUSSION section.  When calibrated correctly and kept very consistent, the arm could pick up boxes consistently from most positions on the belt.  However, the sorting system was never reliable enough to call a success.

Many lessons were learned in the design of industrial-style robotics systems.  It was in the places which received the least amount of hard design work that the largest errors cropped up, which is to be expected.  Solid design work is very important to produce a good product.

# Discussion

The real issues arose from a large error allowed to exist on several components of the system.  We had some trouble inherited from the previous lab which included an inaccurate IR Sensor function.  Our function approximation was not as close as it should have been because we did not calibrate well enough due to poor data-gathering precision.  This caused inconsistencies in the system’s ability to accurately locate several positions on the belt.  When calibrated to locate a close box it did not scale correctly to a far box.  Also, we observed a lot of noise on the sensor line which we were unable to fix.

Another issue arose as a result of design errors.  Our torque sensing function could consistently tell the two weights apart, but the way the measurement was made differed based on different pick-up locations.  Instead, we should have had the arm return to a neutral position and record the torque while the arm makes a controlled, consistent movement.  This meant that the arm could only differentiate between weights from certain locations on the belt.

The conveyor system itself was an issue as well, as the sandpaper belt was quite distorted and a box would move over a significant distance between the measuring sensor and the arm itself.  Repeated attempts to remedy this inconsistency made it difficult to maintain a consistent calibration.

The piece holding the first IR Sensor steady was an issue as well, as it was only held with one screw, allowing the device to make minute rotations and throw off the alignment without being obvious.  This issue would sometimes crop up in the background as adjustments to other parts of the system were being made.

# Appendix A: Final_Project.h

/**

*@file Final_Project.h

*@brief Contains includes, data structures and function protos for the RBE3001 final project

*@date Feb 25, 2010

*@author Adam Panzica

*@author Joel Sutherland

*/

#ifndef FINAL_PROJECT_H_

#define FINAL_PROJECT_H_

#include "ADC.h"

#include "DAC.h"

#include "IR_Sensor.h"

#include "PID.h"

#include "Simple_Serial.h"

#include "portIO.h"

#include "Timer16_Functions.h"

#include "Timer8_Functions.h"

#include "SPI.h"

#include <math.h>

#include <avr\IO.h>

#include <util\delay.h>

#include <avr\interrupt.h>

/**************** SYMBOLIC CONSTANTS *********************/

/** ADC channel for Joint 1*/

#define JOINT1 	0

/** ADC channel for Joint 2*/

#define JOINT2 	2

/** ADC channel for the X axis IR sensor*/

#define IR_X 	3

/** ADC channel for the Y axis IR sensor*/

#define IR_Y 	4

/** ADC channel for the rsense*/

#define RSENSE	1

/** Far cutoff for X direction object detection*/

#define X_MAX 	245

/** Close cutoff for X direction object detection*/

#define X_MIN 	120

/** Value representing an invalid X value (for no object state)*/

#define NO_OBJ 	999

/** Value representing the maximum Y distance */

#define Y_MAX 	240

/** Value representing the minimum Y distance */

#define Y_MIN 	140

/** Value reperesenting the tolerance on the Y target*/

#define Y_TOL 	10

/** Port letter for the electromagnet*/

#define MAGPORT 'B'

/** Pin number for the electromagnet*/

#define MAGPIN	1

/** Value to turn the electromagnet on*/

#define MAGON 	1

/** Value to turn the electromagnet off*/

#define MAGOFF 	0

/** Length of Link 1 in mm **/

#define L_1     70

/** Length of Link 2 in mm **/

#define L_2     70

/** Height of the blocks, in mm */

#define BLOCK_HEIGHT 37

/** Offset between the IR sensor and the base of the robot in the X direction*/

#define X_SHIFT 115

/** Offset between the conveyor belt and the base of the robot in the Y direction*/

#define Y_SHIFT -60

/** X location of the heavy bin */

#define HEAVY_X 270

/** Y location of the heavy bin */

#define HEAVY_Y 80

/** Weight cutoff */

#define HEAVY 13000

/**************** DATA STRUCTURES ***********************/

/**@struct JOINT

* @brief this struct contains all of the data needed to track the state of a joint on the EDUarm

*/

typedef struct JOINT

{

/** Link length (in mm) of the arm link attached to the joint*/

int linkLength;

/** Current joint angle, in degrees */

int jointAngle;

/** Current X location relative to arm base of the end of the link attached to the joint*/

int xLocal;

/** Current Y location relative to arm base of the end of the link attached to the joint*/

int yLocal;

/** ADC value of the motor current for the joint*/

int rsense;

/** Target value for the joint, in ADC values */

int targetVal;

/** Current value of the joint, in ADC values */

int currentVal;

/** Correction value for the joint, in arbitrary PID units */

int correctionVal;

/** LMD output voltage for the joint, in mV*/

int LMDVal;

}JOINT;

/**@union STATE_FLAGS

* @brief Union for holding flags about the current state of the system

*/

typedef union STATE_FLAGS

{

struct

{

/** Flag for the magnet being enabled*/

unsigned _MAGON : 1;

/** Flag for weighing the object*/

unsigned _WEIGH : 1;

/** Flag for object detected*/

unsigned _OBDET : 1;

/** Flag for object in range*/

unsigned _OBINR	: 1;

/** Reserved bits*/

unsigned _RES	: 4;

};

struct

{

unsigned char _FLAGS;

};

}STATE_FLAGS;

/**@struct OBJECT_LOCATION

* @brief data struction for holding the x and y location of objects on the conveyor

*/

typedef struct OBJECT_LOCATION

{

/** X location, relative to object frame, in mm, of a detected object*/

int xLocation;

/** Y location, relative to object frame, in mm, of a detected object*/

int yLocation;

}OBJECT_LOCATION;

/**@struct DATA_PACK

* @brief data structure for passing data back and forth to various functions

*/

typedef struct DATA_PACK

{

/** Pointer to a the first element in an array of joints that the MATLAB program is supposed to operate on. Must be in order staring

* at the joint closest to the base.

*/

JOINT *joint;

/** Pointer to the state flags */

STATE_FLAGS *flags;

/** Pointer to the object Location */

OBJECT_LOCATION *location;

}DATA_PACK;

/**@fn char Check_For_Object(OBJECT_LOCATION *location)

* @brief this function checks to see if there is an object on the conveyer belt

* @returns 1 if object detected, else 0

*/

char Check_For_Object(OBJECT_LOCATION *location);

/**@fn char Check_In_Range(OBJECT_LOCATION *location);

* @brief this function checks to see if the object has reached the pickup area

* @returns 1 if object in range, else 0

*/

char Check_In_Range(OBJECT_LOCATION *location);

/**void Enable_Magnet()

* @brief enables the electro-magnet

*/

void Enable_Magnet();

/**void Disable_Magnet()

* @brief enables the electro-magnet

*/

void Disable_Magnet();

/** char Calc_Inv_Kin(DATA_PACK *data)

* @breif calculates the inverse kinematics for the robot arm target locations

* @param [in] *data pointer to a DATA_PACK struct containing the robot information

* @param [in] block 1 for hard coded block offsets, 0 for arbitrary location passed through the x/y location in the DATA_PACK structure

* @return 1 if the calculation was a success, else 0

* @note also updates the target values for the two joints to the correct values based on the caluclation

*/

char Calc_Inv_Kin(DATA_PACK *data, char block);

/** float Degrees_To_Radians(int degrees)

* @brief converts degrees to radians

* @param int degrees the angle in degrees

* @return the equivalent angle in radians

*/

float Degrees_To_Radians(int degrees);

/** int Radians_To_Degrees(float radians)

* @brief converts radians to degrees

* @param radians the angle in radians

* @return the equivalent angle in degrees

*/

int Radians_To_Degrees(float radians);

/**@fn int main()

* The main function for the final project will implement a simple control system where it runs a PID control on target data recived from Matlab. It will update

* Matlab with the current state of the system at a set frequency, and receive new target data at another set frequency. It will also check to see if there are

* objects on the conveyer and if the are in range of the grabber, which is sent to matlab as well. To summarize, order of operations is as follows:

* 1) Receive new target joint angle data from Matlab

* 2) Check to see if there is an object on the conveyer

* 3) if there was, check to see if it is in grabber range

* 4) If Matlab told it to enable/disable the magnet at the end of the current move, do so if at target values

* 5) Update Matlab with the current state of the robot

* 6) Repeat from step 1

* Background) PID control on the target joint angles

*/

#endif /* FINAL_PROJECT_H_ */

# Appendix B: Final_Project.c

/**

*@file Final_Project.c

*

*@date Feb 28, 2010

*@author Adam Panzica

*@author Joel Sutherland

*/

#include "Final_Project.h"

/*************************** LOCAL FUNCTION PROTOS *************************/

void State_0(DATA_PACK *data);

void State_1(DATA_PACK *data);

void State_2(DATA_PACK *data);

void State_3(DATA_PACK *data);

void State_Machine(char *state, DATA_PACK *data);

char Within(int target, int current);

/*************************** PROGRAM CODE **********************************/

/**Program Entry Point*/

int main()

{

JOINT joints[2];

STATE_FLAGS flags;

OBJECT_LOCATION objLoc;

DATA_PACK dataPack;

char state = 0;

char input;

//initialize pointers in mlMessage

dataPack.flags = &flags;

dataPack.joint = &joints[0];

dataPack.location = &objLoc;

//Set initial values for the state flags

flags._FLAGS = 0;

//Set initial values for the joints

joints[0].linkLength = 200;

joints[0].targetVal = 0;

joints[0].currentVal = 0;

joints[0].LMDVal = 0;

joints[0].correctionVal = 0;

joints[1].linkLength = 195;

joints[1].targetVal = 0;

joints[1].currentVal = 0;

joints[1].LMDVal = 0;

joints[1].correctionVal = 0;

//Set direction registers for debugging use

DDRD = 0xFF;

DDRC = 0x00;

//Initialize serial communications, the ADC, the DAC and PID control

Init_Serial_P(BAUD576, FRM8, STOP1, NOPAR);

ADC_Init_P();

DAC_Initilize();

PID_Initialize();

Serial_Print_String("\r\nMag Pickup Test\r\n");

/*while(1)

{

Serial_Print_String("\r\nIR_X:  ");

Serial_Print_Int(ReadIR(IR_X), 10);

Serial_Print_String("\r\nIR_Y:  ");

Serial_Print_Int(ReadIR(IR_Y), 10);

_delay_ms(200);

}*/

//Calibrate joint pots

Add_Pot_Angle(542, 90, 175, 0, "Joint 1", JOINT1);

Add_Pot_Angle(504, 90, 849, 0, "Joint 2", JOINT2);

//Add PID channels to control the two joints

PID_Add_Channel(&joints[0].targetVal, &joints[0].currentVal, &joints[0].correctionVal, &joints[0].LMDVal, 450, 50, 300, 250);

PID_Add_Channel(&joints[1].targetVal, &joints[1].currentVal, &joints[1].correctionVal, &joints[1].LMDVal, 375, 10, 350, 150);

while(1)

{

joints[0].currentVal = ADC_Get_Value(JOINT1);

joints[1].currentVal = ADC_Get_Value(JOINT2);

joints[0].targetVal = Degrees_To_Pot(30, JOINT1);

joints[1].targetVal = Degrees_To_Pot(60, JOINT2);

State_Machine(&state, &dataPack);

/*Serial_Print_String("\r\nTarget: ");

Serial_Print_Int(Pot_To_Degrees(joints[1].targetVal, JOINT2), 10);

Serial_Print_String("\r\nCurrent: ");

Serial_Print_Int(Pot_To_Degrees(joints[1].currentVal, JOINT2), 10);*/

_delay_ms(100);

}

}

/********************** RANGE DETERMINING FUNCTIONS ****************************/

/**@fn char Check_For_Object(OBJECT_LOCATION *location)

* @brief checks to see if there is an object on the belt, and finds its X position

* @param [in] *location pointer to an OBJECT_LOCATION struct to store object location data to

* @returns 1 if object is detected, else 0

* @note also writes X location data to the OBJECT_LOCATION struct

*/

char Check_For_Object(OBJECT_LOCATION *location)

{

int rangeBuff = 0;

int temp;

char i;

temp = ReadIR(IR_X);

//check to see if there was an object detected

if((temp <= X_MAX)&&(temp >= X_MIN))

{

//take the average of 16 measurements to determine X position

for(i = 0; i<16; i++)rangeBuff+=ReadIR(IR_X);

rangeBuff = rangeBuff/16;

location->xLocation = rangeBuff;

return 1;

}

return 0;

}

/**@fn char Check_In_Range(OBJECT_LOCATION *location)

* @brief checks to see if the object is in grabber range, and finds its Y position

* @param [in] *location pointer to an OBJECT_LOCATION struct to store object location data to

* @returns 1 if object is in range, else 0

* @note also writes Y location data to the OBJECT_LOCATION struct

*/

char Check_In_Range(OBJECT_LOCATION *location)

{

int rangeBuff = 0;

rangeBuff = ReadIR(IR_Y);

location->yLocation = rangeBuff;

if((rangeBuff <= (Y_MAX + Y_TOL)&&(rangeBuff >= (Y_MIN - Y_TOL))))

{

_delay_ms(100);

return 1;

}

return 0;

}

/************************ MAGNET CONTROL FUNCTIONS **************************/

/**Enables the electromagnet*/

void Enable_Magnet()

{

Pin_Set(MAGPORT, MAGPIN, MAGON);

}

/**Disables the electromagnet*/

void Disable_Magnet()

{

Pin_Set(MAGPORT, MAGPIN, MAGOFF);

}

/************************ STATE MACHINE FUNCTIONS ***************************/

/**@fn void State_Machine(char *state, DATA_PACK *data)

* @brief Implements a state transition table for the simple state machine controlling the robot arm

* @param [in] *state Pointer to the state variable

* @param [in] *data Pointer to a DATA_PACK struct containing the robot information

*/

void State_Machine(char *state, DATA_PACK *data)

{

switch(*state)

{

case 0:

State_0(data);

switch(data->flags->_OBDET)

{

case 0:

*state = 0;

break;

case 1:

*state = 1;

break;

default:

*state = 0;

break;

}

break;

case 1:

State_1(data);

switch(data->flags->_OBINR)

{

case 0:

*state = 1;

break;

case 1:

*state = 2;

break;

default:

*state = 0;

break;

}

break;

case 2:

State_2(data);

*state = 3;

break;

case 3:

Serial_Print_String("\r\nPICKING UP OBJECT");

State_3(data);

*state = 0;

break;

default:

Serial_Print_String("\r\nSTATE ERROR!!!!");

break;

}

}

/**@fn void State_0(DATA_PACK *data)

* @brief State 0. Initial startup state and state for after object has been binned

* @param [in] *data pointer to a DATA_PACK struct containing the robot information

*/

void State_0(DATA_PACK *data)

{

JOINT *jptr;

//initialize jptr to allow addressing of the JOINT array in the DATA_PACK structure

jptr = data->joint;

//Move arm to the idle position

jptr[0].targetVal = Degrees_To_Pot(30, JOINT1);

jptr[1].targetVal = Degrees_To_Pot(60, JOINT2);

//Clear object detection/state information

data->flags->_FLAGS = 0;

data->location->xLocation = NO_OBJ;

data->location->yLocation = Y_MAX;

//check for a new object

data->flags->_OBDET = Check_For_Object(data->location);

}

/**@fn void State_1(DATA_PACK *data)

* @brief State 1. Object detected, waiting for it to come into range

* @param [in] *data pointer to a DATA_PACK struct containing the robot information

*/

void State_1(DATA_PACK *data)

{

JOINT *jptr;

//initialize jptr to allow addressing of the JOINT array in the DATA_PACK structure

jptr=data->joint;

//Print out object detection information

Serial_Print_String("\r\nOBJECT DETECTED!\r\n Object X Location: ");

Serial_Print_Int(data->location->xLocation, 10);

//Move arm to the ready position

jptr[0].targetVal = Degrees_To_Pot(30, JOINT1);

jptr[1].targetVal = Degrees_To_Pot(40, JOINT2);

//Check to see if the object is in range

data->flags->_OBINR = Check_In_Range(data->location);

}

/**@fn void State_2(DATA_PACK *data)

* @brief State 2. Object in range, move arm to target and activate magnet

* @param [in] *data pointer to a DATA_PACK struct containing the robot information

*/

void State_2(DATA_PACK *data)

{

JOINT *jptr;

//initialize jptr to allow addressing of the JOINT array in the DATA_PACK structure

jptr=data->joint;

//turn on the electromagnet

Enable_Magnet();

//Move arm to final target location

Calc_Inv_Kin(data, 1);

Serial_Print_String("\r\nMAG ON, MOVING TO TARGET");

Enable_Magnet();

while(!(Within(jptr[0].currentVal, jptr[0].targetVal))||!(Within(jptr[1].currentVal, jptr[1].targetVal)))

{

jptr[0].currentVal = ADC_Get_Value(JOINT1);

jptr[1].currentVal = ADC_Get_Value(JOINT2);

}

//idle waiting for the arm to reach the target

Serial_Print_String("\r\nAT TARGET, PICKING UP OBJECT");

_delay_ms(500);

}

/**@fn void State_3(DATA_PACK *data)

* @brief State 2. Picked up object, Sort into bin

* @param [in] *data pointer to a DATA_PACK struct containing the robot information

*/

void State_3(DATA_PACK *data)

{

JOINT *jptr;

char heavy = 1;

long weightsum = 0;

//initialize jptr to allow addressing of the JOINT array in the DATA_PACK structure

jptr=data->joint;

Serial_Print_String("\r\nSORTING");

//Move arm to the weighing position. Do a running sum on rsense.

//The heavy block will take longer to move, and thus will have a bigger total

jptr[0].targetVal = Degrees_To_Pot(45, JOINT1);

jptr[1].targetVal = Degrees_To_Pot(45, JOINT2);

while(!(Within(jptr[0].currentVal, jptr[0].targetVal))||!(Within(jptr[1].currentVal, jptr[1].targetVal)))

{

jptr[0].currentVal = ADC_Get_Value(JOINT1);

jptr[1].currentVal = ADC_Get_Value(JOINT2);

weightsum += (long)ADC_Get_Value(RSENSE);

Serial_Print_String("\r\nWeight Sum:  ");

Serial_Print_Int(weightsum, 10);

_delay_ms(50);

}

//Sort into correct bin

if(weightsum >= HEAVY) heavy = 1;

else heavy = 0;

switch(heavy)

{

case 0:

jptr[0].targetVal = Degrees_To_Pot(90, JOINT1);

jptr[1].targetVal = Degrees_To_Pot(0, JOINT2);

break;

case 1:

data->location->xLocation = jptr[0].linkLength+jptr[1].linkLength;

data->location->yLocation = 0;

Calc_Inv_Kin(data, 0);

break;

default:

break;

}

//Calculate joint angles to get to the bin and move arm

while(!(Within(jptr[0].currentVal, jptr[0].targetVal))||!(Within(jptr[1].currentVal, jptr[1].targetVal)))

{

jptr[0].currentVal = ADC_Get_Value(JOINT1);

jptr[1].currentVal = ADC_Get_Value(JOINT2);

}

//drop the object

Disable_Magnet();

Serial_Print_String("\r\nOBJECT SORTED, RESETTING");

}

char Calc_Inv_Kin(DATA_PACK *data, char block)

{

JOINT *jptr;

OBJECT_LOCATION *lptr;

int theta1, theta2;

float beta, gamma, delta, targetx, targety;

//Serial_Print_String("\r\nINVERSE KINEMATICS\r\n");

jptr=data->joint;

lptr=data->location;

switch(block)

{

case 0:

targetx = (float)lptr->xLocation;

targety = (float)lptr->yLocation;

break;

case 1:

targetx = (float)(lptr->xLocation+X_SHIFT);

targety = (float)(BLOCK_HEIGHT+Y_SHIFT);

break;

default:

return 0;

}

/*Serial_Print_String("\r\ntargetx: ");

Serial_Print_Int((int)targetx, 10);

Serial_Print_String("\r\ntargety: ");

Serial_Print_Int((int)targety, 10);*/

beta = atan(targety/targetx);

gamma = acos((pow(targetx, (float)2) + pow(targety, (float)2) + pow((float)jptr[0].linkLength,(float)2) - pow((float)jptr[1].linkLength,(float)2))/((float)2*(float)jptr[0].linkLength*sqrt(pow(targetx, (float)2) + pow(targety, (float)2))));

delta = acos((pow((float)jptr[0].linkLength,(float)2)+pow((float)jptr[1].linkLength,(float)2)-(pow(targetx,(float)2)+pow(targety,(float)2)))/((float)2*(float)jptr[0].linkLength*(float)jptr[1].linkLength));

/*Serial_Print_String("\r\nBeta: ");

Serial_Print_Int((int)(beta*100), 10);

Serial_Print_String("\r\nGamma: ");

Serial_Print_Int((int)(gamma*100), 10);*/

theta1 = Radians_To_Degrees(beta + gamma);

theta2 = Radians_To_Degrees(delta) - 90;

/*Serial_Print_String("\r\nTheta1: ");

Serial_Print_Int(theta1, 10);

Serial_Print_String("\r\nTheta2: ");

Serial_Print_Int(theta2, 10);*/

if(!(theta1 >= 0 && theta1 <= 90 && theta2 >= 0 && theta2 <= 90)) return 0;

//Move arm to the proper position

jptr[0].targetVal = Degrees_To_Pot(theta1, JOINT1);

jptr[1].targetVal = Degrees_To_Pot(theta2, JOINT2);

return 1;

}

float Degrees_To_Radians(int degrees)

{

return (float)degrees * (float)3.14159 / (float)180;

}

int Radians_To_Degrees(float radians)

{

return (int)((float)radians*(float)180/(float)3.14159);

}

char Within(int target, int current)

{

if((current <= (target+10))&&(current >= (target-10))) return 1;

return 0;

}